Article XVIII

Off-Street Parking and Loading Regulations

§171-95.

A.  General Off-Street Parking Requirements.

(1)  For all zoning districts, except the Village Center Districts (VC), off-street parking spaces shall be provided for every new structure, the enlargement of an existing structure, or the development of a new land use in accordance with the TABLE OF OFF-STREET PARKING REGULATIONS and other requirements contained herein.

  - Computation of Spaces.  When the computation of required parking or loading spaces results in the requirement of a fractional space, any fraction of one-half (½) or more shall require one (1) space.  An existing structure which is enlarged or an existing use which is extended or expanded after the effective date of this ordinance shall be required to provide off street parking and loading spaces in accordance with the TABLE OF OFF-STREET PARKING REGULATIONS, unless the increase in units or measurements amounts to less than twenty five (25) percent, whether such increase occurs at one time or in successive stages. (Amended Ord 2007- 07, RTCM 3/7/07)

  - Continuance.  Required off-street parking or loading spaces, which, after development, are later designated as, and accepted by the Town for, off-street parking or loading purposes, shall continue to serve the uses or structures to meet these requirements so long as said use or structure remains.

(2)  For all zoning districts, except the Village Center Districts (VC), in cases of a change in use where the existing use (or in cases of vacancy, the next previous use) did not provide for the number of off-street parking spaces required under this Ordinance.  In situations where the proposed use would require a larger number of off-street parking spaces, the proposed use shall only have to provide an additional number of off-street parking spaces equal to the difference between the number required under this ordinance for the proposed use and the number required under this ordinance for the previous use.  Under no circumstances shall this §171-.95. (2) be interpreted to allow a reduction in parking when the required amount of parking spaces for the proposed use would be available on the site. (Amended Ord 2007- 07, RTCM 3/7/07)

(3)  For Village Center Districts (VC) only, no additional off-street parking is required for the continued use or reuse of existing buildings, as long as that use or reuse does not increase the total floor area within the building.  However, off-street parking shall be provided for any new structure, for an enlargement or addition to an existing building, in accordance with the following Table of Off-Street

(4)  Parking Regulations.  For purposes of this ordinance, the replacement of an amount of floor space equal to that in existence at the time of enactment of this ordinance is not considered to be an addition of new space, and is, therefore, exempt from these parking requirements.  Also, the addition of a second floor to one-story buildings existing in the VC Districts at the time of the enactment of this ordinance shall be exempt from these parking requirements so as to encourage the restoration of building heights in the District which are more uniform and consistent with the scale of development which has historically existed.  However, the addition of third, fourth and fifth floors to said one-story buildings in the VC District in existence at the time of adoption of this ordinance shall be subject to the parking requirements as set forth in the Table of Off-Street Parking Regulations.  The Planning Board may waive parking requirements when it can be demonstrated that sufficient public parking is available in the immediate vicinity of the proposed use. (Amended Ord 2007- 07, RTCM 3/7/07)

(5)  The number of off-street parking spaces required to serve added floor space in the Village Center Districts (VC) is shown in parenthesis.  Where no parenthesized number appears, the parking requirement for added floor space in the VC District is the same as that which applies in all other zoning districts.

(6)  Parking shall be provided to serve the parking needs which are generated by a particular use or structure.

B.  General Lot Design.

(1)  Existing Spaces.  Parking or loading spaces being maintained in any district in connection with any existing use on the effective date of this ordinance, or any spaces subsequently provided in accordance with this ordinance, shall not be decreased or any way removed from service to the use originally intended to be served, so long as said use remains, unless a number of parking or loading spaces is constructed elsewhere, such that the total number of spaces conforms to the requirements of the Tables of this Article, provided this regulation shall not require the maintenance of more parking or loading spaces than is required according to the Tables. (Amended Ord 2007- 07, RTCM 3/7/07)

(2)  Location of Loading Spaces.  The loading spaces required for the uses listed in the Table of Off-Street Loading Requirements shall in all cases be on the same lot as the use they are intended to serve.  In no case shall the required loading spaces be part of the area used to satisfy the parking requirements of this ordinance. (Amended Ord 2007- 07, RTCM 3/7/07)

(3)  Location of Parking Spaces.  Required off-street parking spaces shall be provided on the same lot as the principal use they are required to serve.  When practical difficulties exist which prevent their establishment of the same lot, the Planning Board may grant a Special Permit to allow spaces to be on a non-municipal lot, the closest point of which is no further than three hundred (300) feet from the premises to which they are appurtenant.  A Special Permit issued under this provision shall be coterminous with the length of the lease and shall expire if and when the lease for said parking expires.

- Whenever feasible, parking areas shall be located to the side or rear of the structure, and not within the front yard of a structure.

- Whenever feasible, parking areas shall be shared with adjacent businesses.

  - An agreement, lease, deed, contract or easement establishing shared use of a parking facility shall be submitted to, and approved by, the Planning Board.  The approved agreement shall be recorded in the Registry of Deeds prior to the issuance of an occupancy permit for the project.

  - In the event that a shared parking agreement is terminated, those uses with less than the required number of spaces shall notify the Planning Board within fourteen (14) days and do one of the following:

- Provide at least fifty (50) percent of the required parking within one hundred twenty (120) days and provide the remaining required parking within one (1) year following termination of the shared use agreement; or

- Demonstrate to the Planning Board, using a study deemed reliable by the Board, that the available parking is sufficient to accommodate the use’s peak parking demand; or

- Apply for and receive a variance from the Zoning Board of Appeals for less than the required number of spaces.

- Whenever feasible, parking areas shall not be located within twenty (20) feet of the  street line of any lot. (Amended ATM June 11, 2003, Article 11)

- Parking areas shall be designed so that vehicular lights shall be screened or directed away from oncoming traffic.

- Porous pavement may be used on up to twenty (20) percent of the required number of spaces to decrease the amount of impervious surface in the parking area.  No landscaping is required in the porous paving area.

C.  Specific Requirements for Parking Spaces.

(1)  Parking spaces must be at least nine (9) feet by eighteen (18) feet in size, exclusive of maneuvering area and reasonable access.

(2)  All off-street spaces shall have bumper and wheel guards where needed to protect abutting structures, properties or plantings.  Parking areas shall be designed so that parked vehicles do not extend over pedestrian walkways or sidewalks.

(3)  Interior drive widths within parking lots shall be as follows:

- For ninety (90) degree parking – twenty four (24) feet

- For sixty (60) degree parking – eighteen (18) feet

- For forty-five (45) degree parking – thirteen (13) feet

- For thirty (30) degree parking - eleven (11) feet

(4)  It is the obligation of the owner of the property upon which a building or use is located to provide and maintain all required parking and loading spaces as long as such building or use is in existence.

D.  Additional Parking and Loading Space Standards.

(1)  All parking and loading areas shall comply with the following:

- The layout of the parking area shall allow sufficient space for the storage of plowed snow without reducing the number of required parking spaces, unless removal by some other means is provided.

- Any fixture used to illuminate any parking area shall be so arranged as to direct the light away from the street and away from adjoining premises used for residential purposes.

- Parking and loading spaces shall be so arranged as to prohibit backing of vehicles onto any street.

- No portion of a driveways entrance or exit shall be closer than fifty (50) feet to the curb line of an intersecting street nor shall it be closer than fifty (50) feet to any portion of an existing driveway located in a Business or Industrial District.

- Any two driveways leading to or from the same street and from the same lot shall not be within thirty (30) feet of each other at their intersection with the front lot line.

- A driveway's entrance or exit shall not exceed, at its intersection with the front lot line, a width of fifteen (15) feet for single, two and three-family uses; and thirty (30) feet for all other uses.

- The parking area and access driveways thereto shall be graded and drained so as to dispose of all surface water accumulation in accordance with acceptable engineering practices and in accordance with the Town of Palmer street entrance permit standards.

- Except on a farm, not more than one (1) commercial vehicle, and said vehicle shall not exceed a weight of ten thousand (10,000) pounds gross weight, shall be parked or in any way stored on any lot in any Residential (R) District (that is used for residential purposes).  This shall not pertain to a vehicle housed within the confines of a garage or accessory building.

- No private access street or driveway serving a parking lot for nonresidential use shall cross property in a residential district except with a Special Permit granted by the Planning Board under the provisions of Article V.

- No more than one (1) unregistered motor vehicle or trailer, or one (1) registered motor vehicle or trailer not in operating condition (any vehicle that does not have a valid registration legally issued by a governmental authority), and no motor vehicle accessories which are not parts of said one vehicles, may be parked, stored or otherwise placed on a parcel of land in the Town of Palmer without a Special Permit from the Town Manager or Chief of Police.  This section shall not apply to the parking, storage or otherwise placing of unregistered motor vehicles and/or motor vehicle accessories where such parking, storage or placement is in connection with a legally established business selling new and/or used automobiles and trucks, or automotive repair or automobile service stations.  (Amended Ord 2007- 07, RTCM 3/7/07)

- All permitted unregistered motor vehicles and/or motor vehicle accessories shall be screened from the view of the public and from abutting public ways and from abutting properties by being enclosed within a structure or sight impervious fencing or screening.

- A Driveway Permit shall be obtained from the Town Manager, for all new or relocated driveways or parking lots. (Amended Ord 2007- 07, RTCM 3/7/07)

- Any use (drive-ins, etc.), which requires the 'stacking' of vehicles waiting in line, must conform to the standards outlined in Article XV, §171-91.

- There shall not be any business operation for vehicle repair for profit or gasoline or oil service facilities or any repair made to any motor vehicles, except on a lot occupied by a permitted automotive use.  Any gasoline or oil facilities shall be at least twenty-five (25) feet from any lot line.

- There shall not be any storage of materials or equipment or display of merchandise within parking areas except as part of approved building operations.

(2)  In addition to the above, all parking and loading areas containing over five (5) spaces, including automotive and drive-in establishments of all types shall either be contained within structures or shall also comply with the following:

- The area shall be effectively screened with suitable planting or sight impervious fencing on each side which adjoins or faces the side or rear lot line of a lot situated in any Residential "R" District or any lot containing a residential use.

- The area and access driveways thereto shall be surfaced with bituminous concrete or cement concrete material.  The Planning Board may allow construction of an alternative all-weather surface if it can be demonstrated to the satisfaction of the Board that such surface will be durable, dustless and continuously maintained.  However, bituminous or cement concrete must be used in all areas of a ten (10%) percent slope or greater.  The location of spaces shall be suitably marked by painted lines or other appropriate markings.

- A substantial bumper of masonry, steel, or heavy timber, or a concrete curb or berm curb which is backed, shall be placed at the edge of surfaced areas except driveways in order to protect abutting structures, properties and sidewalks and screening materials.  In addition, a minimum four (4) foot wide sidewalk is required to separate spaces from the building that they serve.

- In all Zoning Districts except the General Business (GB), the Planning Board may                                                grant a Special Permit to allow the reduction of the parking space requirements to eighty (80) percent of that required in the Table of Off-Street Parking Regulations where conditions unique to the use will reasonably justify such a reduction, provided that a greater percentage reduction may be allowed where joint use of the same spaces by two or more uses or establishments is justifiable by virtue of the fact that the uses or establishments generate peak demand at substantially different time periods.

- In the design of parking lots serving uses located in Business or Industrial Districts which provide more than seventy-five (75) parking spaces, the expanse of pavement shall be interrupted by separating rows of parking spaces from each other and from driveways by using planting strips which may also contain pedestrian sidewalks at least six (6) feet in width combined.  Provision of these required planting strips shall take into account the need to store snow, the need to locate light poles, the need to allow safe pedestrian movement, the need to maximize emergency access, and the need to separate different traffic movements.  Any modification to a particular parking lot which caused the lot to exceed the seventy-five (75) car standard shall cause the provision of planting strips to be required in the entire lot.  All proposals to construct or modify such parking lots shall be reviewed by the Planning Board in light of the requirements of this Section.

- Fire lanes or emergency access points required for buildings or other structures shall be protected from unauthorized parking through the provision of curbs, mountable barriers, landscaped areas or such other improvements subject to the approval of the Fire Chief and Chief of Police, which in turn meets the objective of precluding parking in the restricted area.

E.  Shared Parking

1) Shared On-Site Parking

To implement shared on-site parking, the applicant shall provided analyses as part of Site Plan Review to demonstrate that proposed uses are either competing or non-competing.

a) Non-competing Uses. In mixed-use developments, applicants may propose a reduction in parking requirements based on an analysis of peak demands for non-competing uses. Up to [75%] of the requirements for the predominant use may be waived by the Planning Board if the applicant can demonstrate that the peak demands for two uses do not overlap. An applicant may use the latest peak demand analyses published by the Institute of Traffic Engineers (ITE) or other source acceptable to the [Planning Board].

b) Competing Uses. In mixed-use developments, applicants may propose a reduction in parking requirements where peak demands do overlap. In these cases, the Planning Board may reduce the parking requirements of the predominant use by up to [30%].

2) Off-Site Parking

Separate from, or in conjunction with Shared Parking provisions, an applicant may use off-site parking to satisfy their parking requirements. As part of Site Plan Review, the applicant shall provide the necessary information to comply with the following standards:

a) Off-site parking shall be within [five hundred (500)] feet of the property for which it is being requested.

b) Off-site parking may only be provided if the off-site lot has an excess number of spaces or if the applicant can demonstrate that the on-site and off-site uses have non-competing peak demands.

c) The amount of required parking spaces being reduced on-site shall be equal to the amount being provided off-site and can account for up to 100% of the minimum required on-site parking.

d) Off-site parking spaces provided by a separate private property owner shall be subject to a legally binding agreement that will be presented to the Planning Board during the Site Plan Review process or as a condition of approval. If the conditions for shared parking become null and void and the shared parking arrangement is discontinued, this will constitute a zoning violation for any use approved expressly with shared parking. The applicant or property owner must then provide written notification of the change to the Zoning Enforcement Official and, within 60 days of that notice, provide a remedy satisfactory to the Commission to provide adequate parking.

e) Off-site parking provided by means of a public parking facility shall be limited to [50%] of the overall parking requirement [for daytime peak uses].

f) On-street parking spaces that [intersect or] are completely contained within the frontage of the property may be counted toward the minimum parking requirements.

g) Uses sharing a parking facility shall provide for safe, convenient walking between uses and parking, including safe, well marked pedestrian crossings, signage, and adequate lighting.

F. Parking Lot Design

1) Compact Cars

Applicant may design up to 30% of their parking spaces for compact cars in accordance with the dimensions listed in Section 7.B of this bylaw. Compact car spaces shall be grouped together to the greatest possible extent in areas clearly designated for compact cars. Parking lots shall have a system of signs beginning at the entrance that clearly indicates the location of compact car spaces.

2) Parking Space and Travel Lane Dimensions

For the purposes of this bylaw, minimum parking space width shall be measured perpendicular to the center line of the parking space. For standard cars the minimum parking space width shall be nine (9) feet. For compact cars, the minimum parking space width shall be eight (8) feet. Travel lanes and associated module widths shall conform to the following minimum standards:

| Parking Angle | Parking Stall Width (1) |  | Travel Land (one way) |  | Travel Lane (two way) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Standard Space | Compact Car | Standard Space | Compact Car | Standard Space | Compact Car |
| Parallel | 9’ | 8’ | 12’ | 12’ | 24’ | 22’ |
| 45 degrees | 18’ | 16’ | 14’ | 12’ | 24’ | 22’ |
| 60 degrees | 21’ | 17.5’ | 16’ | 14’ | 24’ | 22’ |
| 75 degrees | 22’ | 19’ | 19’ | 16’ | 24’ | 22’ |
| 90 degrees | 20’ | 17’ | 22’ | 19’ | 24’ | 22’ |

1 Measured from the inner most point on the parking space centerline perpendicular to the edge of the Travel Lane.

G. Landscaping Standards for Parking Lot Stormwater Management

Landscaping is required for all parking lots and may be designed in one of two ways as related to stormwater management pursuant to the requirements in Section 4: 1) Low Impact Development (LID) Parking Area Design; or 2) Conventional Parking Area Design. LID Landscaping Plans shall denote a drainage design where [75% or more] ofthe [first half inch] of stormwater runoff from impervious surfaces is treated for water quality by a combination of LID techniques in accordance with the most recent version of the Massachusetts DEP Stormwater Management Manual. Conventional Parking Area Design shall denote a parking lot landscape design that does not meet the criteria for LID Parking Area Design.

Acceptable LID techniques shall include vegetated swales, rain gardens or bioretention facilities, permeable pavers, infiltration facilities and constructed wetlands. Cisterns and grey water systems that recycle stormwater runoff may also be included in these calculations.

For parking areas that will contain fewer than [ten (10)] spaces, compliance with the design standards set forth in this bylaw shall be determined by the Zoning Enforcement Officer.

1)  Conventional Parking Area Design Standards

The landscaping requirements in this section are intended to provide a baseline set of standards toward reducing the visual impacts of large areas of pavement, improving the overall environment or parking areas by providing areas for shade and heat reduction, and enhancing the overall aesthetic appeal of parking areas. The following standards shall apply to all Conventional Parking Lot Design as defined in this bylaw.

a) Amount. Developments with proposed parking areas of [ten (10)] spaces or more shall provide a minimum of 10% of the total parking area as landscaped open space.

b) Buffers. Landscaping shall be required between non-residential uses or mixed use developments and existing or future residential development areas. Buffer zones shall be a minimum of [twelve (12) feet] in width and shall substantively screen the site from view through the use of evergreen vegetation at least six feet in height. Fences may be used as part of screening but shall not include chain link fences. These requirements shall not apply to non-residential or mixed use development that are designed to integrate existing or future neighboring residences into the site through the use of walkways, bicycle paths or other pedestrian amenities.

c) Parking Lot Entrances. Parking lot entrances shall be landscaped minimally with a combination of trees and shrubs. These areas may also be used for signage in compliance with [INSERT REFERENCE TO SIGNAGE SECTION OF BYLAW]. No trees or shrubs shall be planted in a way to obstruct sight lines of motorists.

d) Parking Aisles. The ends of parking aisles that are more than [fifteen (15) spaces] in length shall incorporate landscape islands at either end of the row. Where the length of parking aisles exceeds [twenty-five (25)] spaces, an intermediary landscaped island shall be installed a regular intervals. This interval shall not be more than every [thirteen (13)] spaces. Landscape islands used at the end of parking aisles shall enclose. The width of landscaped islands at their ends shall not be less than [four (4)] feet and not less than [eight (8)] feet at their midpoint.

e) Plant Selection. No tree, shrub or plant shall be proposed for use within a parking area that has been identified as an Invasive Species by the Massachusetts Plant Advisory Group in the latest version of The Evaluation of Non-Native Plant Species for Invasiveness in Massachusetts (with annotated list), has been identified as invasive or banned on the Massachusetts Prohibited Plant List as periodically updated by the Massachusetts Department of Agricultural Resources, or in any other reputable scientific publication that may be acceptable to the Board. All size and location design elements shall comply with the following specifications: a) Shade or canopy trees shall be three (3) inches DBH with a height of not less than twelve (12) feet above grade; b) Small or minor shade trees shall be two and one-half (2.5) inches DBH with a height of not less than nine (9) feet above grade; c) Ornamental or flowering fruit trees shall be two (2) inches DBH with a height of not less than seven (7) feet above grade; d) Evergreen trees used for screening shall be not less than six (6) feet in height above grade. Fencing may be used in conjunction with vegetated screening [but chain link fence shall not be allowed]; e) Shrubs shall be not less than one and one-half (1.5) feet in height above grade. f) Turf may be used but shall not be installed in strips less than six (6) feet in width.

H.  LID Parking Area Design Standards

The purpose of these standards is provide the Zoning Enforcement Officer or the parties involved with Site Plan Review the opportunity to review plans for a lower impact approach to managing stormwater in parking areas. The following information is therefore required of an applicant choosing to treat any portion of a parking lot with LID stormwater management techniques. This informationshall be prepared by a Massachusetts registered Professional Engineer and shall comply with the design and implementation guidelines provided in the latest version of the Massachusetts DEP Stormwater Management Manual. Where portions of the parking lot are not using acceptable LID techniques, the standards for Conventional Parking Lot Design in Section 8.A shall apply.

1) Delineation of all drainage areas inclusive of areas outside of the parking envelope that will contribute stormwater runoff to the parking area;

2) Proposed topography at two-foot contour intervals;

3) Site Plan showing drainage pathways and locations of proposed BMPs;

4) Typical profiles of BMPs;

5) Sizing calculations for BMPs that demonstrate adequate conveyance and/or water quality treatment of the [first half inch of stormwater runoff from impervious surfaces]; 6) Sizing calculations for BMPs that illustrating proposed management of runoff resulting from 2-year, 10-year, and 100-year event;

7) List of plantings associated with vegetated BMPs;

8) Location of areas reserved for snow storage;

9) Location of any screening between residential and non-residential properties. Buffer zones shall be a minimum of [six (6) feet] in width and shall substantively screen the site from view through the use of evergreen vegetation at least six feet in height. Fences may be used as part of screening but shall not include chain link fences. These requirements shall not apply to non-residential or mixed use development that are designed to integrate existing or future neighboring residences into the site through the use of walkways, bicycle paths or other pedestrian amenities.

10) Location of test pits, depth to seasonal high ground water and soil percolation rates for those areas designated for recharge;

11) Schematic diagrams of any gray water or cistern systems proposed for the parking area;

12) An Operation and Maintenance (O&M) Plan shall be submitted by the applicant to the Zoning Enforcement Officer or the [Planning Board] that conforms to the standards for O&M Plans detailed in the most recent version of the Massachusetts DEP Stormwater Management Manual. The LID requirements listed above are designed to mirror the Massachusetts

§171-96.  TABLE OF OFF-STREET PARKING REGULATIONS

| Section 171-96: Table of Off Street Parking Requirements |  |
| --- | --- |
| Use | Number of Off Street Parking Spaces (When it is deemed that additional parking spaces are required in a VC District, the number in parentheses (xx) shall be used) |
| Dwelling, one, two, and three family units and Townhouses and condominiums | Two per unit (1.0 per unit) |
| Dwelling, multi-family | Two per dwelling unit, except housing for the elderly, in which case, it shall be one for each two dwelling units (1.0 per two units) |
| Lodging house, and similar type of group activities. | One per rental or sleeping unit. Any bedroom or group of two beds in a single room constitutes a sleeping unit. For lodging houses only, the Planning Board may issue a Special Permit to allow a reduction in the required number of parking spaces to one per every two rental or sleeping units. |
| Theater, gymnasium, auditorium, church or similar place of public assembly, with seating facilities | One for each three seats of total seating capacity (one for each six seats) |
| Automobile retail and ser vice establishment, and other retail and service establishment utilizing either indoor or outdoor display areas, which are unusually extensive in relation to customer traffic | One per 500 square feet of gross floor space. In the case of outdoor display areas, one for each vehicle allowed by the license for the site |
| Hotel, motel, tourist court | One for each sleeping room plus one for each 400 sq. ft. of public meeting area and restaurant space |
| Drive-in eating establishment | One per 30 square feet of gross floor area |
| Drive-in establishment | One per 100 square feet of gross floor area |
| Tourist home/bed and breakfast | Two spaces, plus one additional space for each rooming unit. Bed and breakfast: Maximum: 1.2 spaces per guest room or suite Minimum: 1 space per guest room or suite |
| Establishments selling foods prepared on premises, where consumption is primarily off the premises | Three for each four seats of seating capacity, plus one for each 100 square feet of kitchen area and waiting area |
| Sit-down restaurants, lounges, bars, and nightclubs breakfast facility | One for each four seats of total seating capacity, plus one per each 300 square feet of gross kitchen area (one for each four seats of total seating capacity, plus one per each 500 square feet of gross kitchen area) Restaurants: Maximum: 10 per 1000 square feet GFA. Minimum: 6 per 1000 square feet GFA |
| Commercial, retail and personal service establishments (excluding convenience stores) | One per each 200 square feet of gross floor area (1.0 per 500 square feet) Excluding all areas used for storage and/or utility uses. Except this may be reduced by Planning Board to one per 250 square feet for businesses with over 20,000 square foot floor areas. Large-scale Retail: Maximum: 4 per 100 square feet GFA. Minimum: 2 per 1000 square feet GFA Free Standing Retail: Maximum: 3 per 1000 square feet GFA Minimum: 1 per 1000 square feet GFA Shopping Centers: Maximum: 4 per 1000 square feet GFA Minimum: 3 per 1000 square feet GFA |
| Convenience Stores | One per 150 square feet. |
| Miscellaneous professional and business offices, including banks, insurance and real estate establishments | One per each 200 square feet of gross floor area (1.0 per 500 square feet) Excluding all areas used for storage and/or utility uses. Bank: Maximum: 3 per 1000 square feet GFA. Minimum: 2 per 1000 square feet GFA |
| Medical/dentist office building | One per each 200 square feet of gross floor area (1.0 per 400 square feet) Excluding all areas used for storage and/or utility uses. General office Building: Maximum: 4 per 1000 square feet GFA. Minimum: 2 per 1000 square feet GFA Medical Office Building: Maximum: 8 per 1000 square feet GFA Minimum: 2 per 1000 square feet GFA |
| Wholesale establishment | One per each 1,000 square feet gross floor space |
| Warehouse or storage establishment | One per two employees on the two largest shifts combined |
| Manufacturing or industrial establishment | One per each 800 square feet of gross floor space OR .75 per each employee of the combined employment of the two largest successive shifts, whichever is the larger Industrial plant: Maximum: 2 per 1000 square feet GFA. Minimum: 1 per 1000 square feet GFA |
| Hospital | One and one-half spaces per bed at design capacity |
| Business, trade, or industrial school or college | One for each 200 square feet of gross floor area in classrooms, including space for the gymnasium or the auditorium whichever has the larger capacity at one space per (3) three seats. |
| Country club | One for each four regular club members, plus one space for each employee |
| YMCA, community facility (Town building, recreation, etc.) | One per each 400 square feet of gross floor space (none) |
| Libraries and museums | One per each 750 square feet of gross floor space (none) Maximum: 2 per 1000 square feet GFA Minimum: 1 per 1000 square feet GFA |
| Transportation terminal | One for each 500 square feet of gross floor area |
| Public utility | 1. One for each 300 square feet of gross floor area devoted to office use. 2. One for each 800 square feet of gross floor area per other use |
| Funeral parlors | One for each four seats of seating capacity. |
| Car washing facility (automatic or conveyor) | One per employee plus stacking spaces equivalent to 5 times the maximum capacity of the facility |
| Mixed use | Sum of various uses computed separately |
| Elementary and Junior High School | One space for each teacher and employee, including space for the gymnasium or the auditorium whichever has the larger capacity at one space per three (3) seats. |
| High School | One space for each teacher and employee, plus one space per each four students, including space for the gymnasium or the auditorium, whichever has the larger capacity at one space per three (3) seats. Public and Private Educational Institutions: Maximum: 1 space per 3 seats in the classroom Minimum: 1 space per 5 seats in the classroom |
| Large scale conference and Entertainment centers | 1 per three (3) seats or 1 per three (3) persons attending during peak hours. |
| Flea Market | Four (4) spaces per vendor |
| Outdoor recreational use | 1 per three (3) persons attending during peak hours. (Added ATM – June 16, 2003 – Article 11) |
| Nursing Home | Maximum: 3 per 1000 square feet GFA Minimum: 2 per 1000 square feet GFA |
| Day Care Centers | Maximum: 1 space per 4 children at max. capacity Minimum: 1 space per 8 children at max. capacity |
| Personal Services | Maximum: 2 per 1000 square feet GFA Minimum: 2 per 1000 square feet GFA |
| Churches and Places of Worship | Maximum: 1 space per 3 seats in portion of building used for services Minimum: 1 space per 5 seats in portion of building used for services |
| Social, Fraternal Clubs and Organizations | Maximum: 4 per 1000 square feet GFA Minimum: 3 per 1000 square feet GFA |
| Any use permitted by this ordinance not interpreted to be covered by this schedule | Closest similar use as shall be determined by the Zoning Enforcement Officer |

§171-97.  Handicap Parking.

A.  All parking areas shall be provide handicapped accessible parking spaces, as required by the federal American with Disabilities Act (ADA), and as specified in the table below, except for the following uses which are specifically exempted in ADA requirements:

(1)  Owner occupied buildings with no more than four (4) units.

(2)  Single family homes, not owner occupied, sold or rented without the use of a broker.

(3)  Housing operated by religious organizations and private clubs that limit occupancy to members.

B.  Parking for the handicapped shall be provided at the rate specified in the table below:

| Total Spaces in Lot | 1-25 | 26-50 | 51-75 | 76-100 | 101-150 | 151-200 | 201-300 | 301-400 | 401-500 | 501-1000 | 1000 & Over |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum Number of Accessible spaces | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 2% of total | 20 plus 1 for each 100 over 1000 |

C.   One (1) van accessible handicap space is required for each eight (8) standard handicap spaces required.  At least one (1) van accessible space is required for each use.

D.   Accessible spaces shall be eight feet (8’-0”) wide, with an adjacent access aisle five feet (5’-0”) wide, and shall be marked with signs and pavement paint.  One (1) in every eight (8) accessible spaces shall have an access aisle eight feet (8’-0”)  (rather than five feet (5’-0”)) and shall be signed “Van Accessible”.

§171-.98.  Off-Street Loading and Unloading Requirements.

A.  For every building hereafter erected for Retail and Service Commercial, Wholesale, Transportation and Industrial, and Community Facility use as specified in the Table of Use Regulations and for every such use hereinafter established in an existing building or area, the off-street loading and unloading requirements presented in the Table of Off-Street Loading Regulations shall apply.

§ 171-99.  Table of Off Street Loading Regulations.

| Type of Use | First Loading Space | Second Loading Space | Each Additional Space |
| --- | --- | --- | --- |
| Retail Trade | 20,000 sq. ft. | 60,000 sq. ft. | One for 60,000 sq. ft. of GFA |
| Wholesale Distribution | 20,000 sq. ft. | 60,000 sq. ft. | One for 60,000 sq. ft. of GFA |
| Hotels and Motels | 20,000 sq. ft. | 100,000 sq. ft. | One for each 100,000 sq. ft. of GFA |
| Business Services | 75,000 sq. ft. | 200,000 sq. ft. | One for each 250,000 sq. ft. of GFA |
| Other Services | 75,000 sq. ft. | 200,000 sq. ft. | One for each 250,000 sq. ft. of GFA |
| *Hospital | 20,000 sq. ft. | 100,000 sq. ft. | One for each 100,000 sq. ft. of GFA |
| Manufacturing | 15,000 sq. ft. | 40,000 sq. ft. | One for each 60,000 sq. ft. |
| Motor Freight Terminal and Warehousing | 15,000 sq. ft. | 40,000 sq. ft. | One for each 60,000 sq. ft. |
| Community Facility, School, Church, Town Building, Recreation Area, etc. | 75,000 sq. ft. | 150,000 sq. ft. | One for each 200,000 sq. ft. |
| Public Utility Establishment with over 5,000 sq. ft. of GFA | 75,000 sq. ft. | 150,000 sq. ft. | One for each 200,000 sq. ft. |

*Space used for ambulance receiving at a hospital is not used to meet these loading requirements.